FAULT DETECTION ANALYSIS IN SIMULATED DETECTOR READOUT ELECTRONICS USING PARITY AND CRC-8

1. ABSTRACT

Reliable data transmission is essential in digital detector readout systems used in scientific instrumentation. Bit errors can corrupt measurement data and affect the reliability of subsequent analysis. This project presents a simulation based comparison of parity checking and Cyclic Redundancy Check (CRC-8) for detecting faults in a 16-bit detector readout word. A Python based fault-injection experiment evaluates all 65,535 nonzero fault masks for a fixed data word, 0xA53C. The results show that parity detected 32,768 fault masks, corresponding to 50.001%, whereas CRC-8 detected 65,280 masks, corresponding to 99.611%. The experiment demonstrates the stronger detection capability of CRC-8 for the tested error patterns. Verilog is also used to model digital readout behaviour and fault detection. The study highlights the importance of validating error-detection mechanisms before using them in reliable electronic systems.

2. INTRODUCTION

Digital detector readout electronics convert detector measurements into digital data for processing and analysis. During transmission or processing, bit errors may occur because of electronic disturbances, signal-integrity problems, or hardware faults.

Error-detection techniques help identify corrupted data. Parity checking is simple and requires only one check bit, but it cannot detect errors that change an even number of bits. CRC techniques use polynomial-based calculations to detect a broader range of error patterns.

This project compares parity and CRC-8 through simulated fault injection and analyses their detection performance.

3. PROBLEM STATEMENT

A digital readout system needs a reliable way to identify corrupted data. Simple parity checking may fail to detect certain multi-bit errors. Therefore, a comparison is required to determine how effectively parity and CRC-8 detect different fault patterns in a 16-bit data word.

4. OBJECTIVES

- Model a 16-bit digital detector readout using Verilog.

- Simulate bit-flip faults using fault masks.

- Implement parity and CRC-8 calculations in Python.

- Evaluate both methods across all nonzero 16-bit fault masks for a fixed data word.

- Compare detection rates and identify undetected fault patterns.

5. PROPOSED SYSTEM

The proposed system uses a digital data word as the input to a simulated readout channel. A fault mask is applied using a bitwise XOR operation to generate potentially corrupted data.

The original data word is used to calculate reference parity and CRC-8 values. The received data is checked against these references. The experiment records whether each method detects the injected error.

SYSTEM FLOW:

Original 16-bit Data

Calculate Reference Parity and CRC-8

Inject Bit-Flip Fault

Check Received Data

Record Detection Results

Compare Performance

6. TOOLS AND TECHNOLOGIES

- Verilog/SystemVerilog: Digital readout and fault-injection model.

- EDA Playground: Online HDL simulation.

- Python: Fault-mask generation and detection-rate calculations.

- Google Colab: Execution of Python experiments and visualization.

- Matplotlib: Graph generation for result analysis.

7. METHODOLOGY

7.1 Data selection

A fixed 16-bit data word, 0xA53C, is selected for the experiment.

7.2 Fault injection

A fault mask is generated for every integer from 1 through 65,535. The received word is calculated as:

received_data = original_data ⊕fault_mask

A set bit in the fault mask flips the corresponding data bit.

7.3 Parity checking

The parity of the original and received data words is calculated and compared. If the parity differs, the fault is detected.

7.4 CRC-8 checking

CRC-8 is calculated using polynomial 0x07, initial value 0x00, MSB-first processing, and no final XOR. The CRC of the received data is compared with the reference CRC of the original data.

7.5 Performance measurement

For each method, the experiment counts detected and undetected fault masks and calculates the detection rate:

Detection rate = Detected fault masksTotal fault masks× 100

8. EXPERIMENTAL RESULTS

The following results were obtained from the Python fault-injection experiment.

| Parameter | Parity | CRC-8 |
| --- | --- | --- |
| Total nonzero fault masks | 65,535 | 65,535 |
| Detected fault masks | 32,768 | 65,280 |
| Undetected fault masks | 32,767 | 255 |
| Detection rate | 50.001% | 99.611% |

CRC-8 detected 32,512 fault masks that parity missed. These results indicate that CRC-8 performed substantially better than parity for the tested fault masks.

9. RESULT ANALYSIS

Parity detects errors that change the parity of the data word, including all odd-weight bit-flip masks. However, errors that flip an even number of bits preserve parity and are not detected.

CRC-8 detects a much wider range of error patterns because it uses polynomial-based redundancy. Nevertheless, CRC-8 is not guaranteed to detect every possible error. Some nonzero fault masks produce the same CRC as the original data.

The reported results apply to the selected CRC implementation, fixed data word, and exhaustive set of nonzero data-bit fault masks. They do not represent every possible detector fault or a measured real-world hardware failure rate.

10. APPLICATIONS

- Digital detector readout systems.

- Scientific instrumentation and data-acquisition electronics.

- Communication interfaces between digital modules.

- Embedded systems requiring data-integrity checks.

- Educational studies of fault injection and error detection.

11. LIMITATIONS

- The exhaustive experiment uses one fixed 16-bit data word.

- The study models data-bit flips rather than all possible electronic hardware faults.

- The reference check values are assumed to remain intact.

- The Python experiment does not, by itself, establish that the complete Verilog implementation behaves identically.

- CRC-8 can miss certain error patterns and should not be treated as perfect fault detection.

12. FUTURE SCOPE

Future work can extend the experiment to multiple data words, different CRC polynomials, burst-error patterns, corrupted check bits, and additional fault types. The Python results can also be cross-validated against the Verilog implementation using automated test cases. More advanced work could investigate fault-campaign efficiency and methods for identifying fault patterns that remain undetected.

13. CONCLUSION

This project presents a simulation-based evaluation of parity and CRC-8 for detecting bit-flip faults in a 16-bit digital readout word. In the exhaustive Python experiment, parity detected 50.001% of the tested fault masks, while CRC-8 detected 99.611%. The findings demonstrate the limitations of parity checking and the improved detection performance of CRC-8 for the chosen test conditions. The work provides a foundation for further study of data integrity and fault analysis in digital electronics.

14. REFERENCES

- W. W. Peterson and D. T. Brown, “Cyclic Codes for Error Detection,” Proceedings of the IRE, vol. 49, no. 1, pp. 228–235, 1961.

- W. W. Peterson and E. J. Weldon Jr., Error-Correcting Codes, 2nd ed., MIT Press, 1972.

- IEEE, IEEE Standard for SystemVerilog—Unified Hardware Design, Specification, and Verification Language, IEEE Std 1800.